# Comparability Assessment Report

# Product: mAb-DocTest-01

This comparability study evaluates the impact of a manufacturing process change (scale-up from 2000L to 5000L bioreactor) on the monoclonal antibody mAb-DocTest-01. The post-approval commercial product was assessed across physicochemical, purity, potency, and stability attributes.

# Comparability Data

| Attribute | Category | Pre-Change | Post-Change | Specification | Unit |
| --- | --- | --- | --- | --- | --- |
| SEC Monomer | Purity | 98.5 | 98.2 | >=95.0 | % |
| SEC Aggregate | Purity | 0.8 | 1.0 | <=2.0 | % |
| CEX Main Peak | Physicochemical | 65.2 | 64.8 | 55.0-75.0 | % |
| Potency (Bioassay) | Potency | 102.0 | 99.5 | 80.0-120.0 | %RP |
| Osmolality | Physicochemical | 290.0 | 292.0 | 270.0-330.0 | mOsm/kg |
| Endotoxin | Purity | 0.05 | 0.04 | <=0.5 | EU/mL |

# Conclusion

All tested attributes met acceptance criteria. The manufacturing scale-up from 2000L to 5000L did not adversely impact product quality.