# CTD Module 3 - Quality

# 3.2.S.1 General Information (Drug Substance)

The drug substance is a monoclonal antibody (mAb) produced by recombinant DNA technology in Chinese Hamster Ovary (CHO) cells. The INN is Examplemab.

Molecular formula: Approximately 145 kDa IgG1 monoclonal antibody. The molecule targets the CD20 antigen on B lymphocytes.

# 3.2.S.2 Manufacture (Drug Substance)

The drug substance is manufactured at BioFacility Corp., Site A. The manufacturing process consists of upstream cell culture, harvest, and downstream purification including Protein A chromatography, ion exchange, and viral inactivation steps.

| Parameter | Target | Actual |
| --- | --- | --- |
| Cell Culture Duration | 14 days | 14 days |
| Harvest Titer | >2.0 g/L | 2.8 g/L |
| Protein A Yield | >80% | 92% |

# 3.2.S.3 Characterisation (Drug Substance)

Comprehensive characterization was performed per ICH Q6B including primary structure confirmation by peptide mapping (LC-MS/MS), higher-order structure by CD and DSC, glycan profiling by HILIC-MS, charge variants by icIEF, and size variants by SEC-HPLC.

Reference standard lot: RS-2024-001. HMW: 1.2%. Main charge peak: 65.3%. Afucosylation: 8.5%. Relative potency: 98%.

| Attribute | Method | Result | Unit |
| --- | --- | --- | --- |
| HMW (Aggregates) | SEC-HPLC | 1.2 | % |
| Monomer Purity | SEC-HPLC | 97.5 | % |
| Main Charge Peak | icIEF | 65.3 | % |
| Afucosylation | HILIC-MS | 8.5 | % |
| Relative Potency | Cell-based assay | 98 | % |
| Binding Affinity (Kd) | SPR/Biacore | 0.15 | nM |

# 3.2.S.4 Control of Drug Substance

Method validation was performed per ICH Q2(R2) for all release testing methods. Specificity, linearity, accuracy, precision (repeatability and intermediate), range, LOD, LOQ, and robustness were assessed.

SE-HPLC Method Validation: Recovery: 99.5%. RSD: 1.2%. R2: 0.9998. LOD: 0.05%. LOQ: 0.1%.

| Validation Parameter | Result | Acceptance Criteria |
| --- | --- | --- |
| Accuracy (Recovery) | 99.5% | 98.0-102.0% |
| Precision (%RSD) | 1.2% | <=2.0% |
| Linearity (R2) | 0.9998 | >=0.999 |
| LOQ | 0.1% | Report value |

# 3.2.S.5 Reference Standards (Drug Substance)

The primary reference standard (lot RS-2024-001) was fully characterized and qualified. Traceability to WHO International Standard established.

# 3.2.S.6 Container Closure System (Drug Substance)

Drug substance is stored in 2L polycarbonate bottles with HDPE screw caps. Container closure integrity testing per USP <1207> confirmed adequate seal.

# 3.2.S.7 Stability (Drug Substance)

Stability studies were conducted per ICH Q1A/Q5C at long-term (5C), accelerated (25C/60%RH), and stress (40C/75%RH) conditions. Proposed shelf-life of 24 months at 2-8 degrees C.

| Attribute | Condition | T=0 | 3M | 6M | 12M |
| --- | --- | --- | --- | --- | --- |
| Purity (SEC) | 5C | 97.5 | 97.3 | 97.1 | 96.8 |
| HMW | 5C | 1.2 | 1.3 | 1.4 | 1.6 |
| Potency | 5C | 98 | 97 | 96 | 95 |
| Purity (SEC) | 40C/75RH | 97.5 | 95.0 | 92.1 | 88.5 |

# 3.2.P.1 Description and Composition (Drug Product)

The drug product is a sterile, preservative-free solution for IV infusion. Each vial contains 100 mg of Examplemab in 10 mL (10 mg/mL) formulated in 20 mM histidine, 240 mM trehalose, 0.02% polysorbate 80, pH 6.0.

# 3.2.P.2 Pharmaceutical Development

Formulation development studies optimized buffer, pH, surfactant, and tonicity agent to maximize stability. Forced degradation studies confirmed the stability-indicating nature of all analytical methods.

# 3.2.P.3 Manufacture (Drug Product)

Drug product manufacture includes thawing of drug substance, dilution, sterile filtration (0.22 um), aseptic fill into glass vials, stoppering, and capping. Process validation completed for 3 consecutive batches.

# 3.2.P.4 Control of Excipients

All excipients comply with USP/NF or Ph. Eur. monographs. Polysorbate 80 is compendial grade. Histidine and trehalose meet USP specifications.

# 3.2.P.5 Control of Drug Product

Release testing specifications and validated methods for drug product. Method validation per ICH Q2(R2). Specificity, linearity, accuracy, precision, and robustness demonstrated for all methods.

| Test | Method | Acceptance Criteria |
| --- | --- | --- |
| Appearance | Visual | Clear, colorless to slightly yellow |
| pH | Potentiometry | 5.7 - 6.3 |
| Protein Concentration | UV A280 | 9.0 - 11.0 mg/mL |
| Purity (SEC) | SE-HPLC | >= 95.0% |
| Potency | Cell-based assay | 80 - 120% |

# 3.2.P.6 Reference Standards (Drug Product)

The same primary reference standard (lot RS-2024-001) is used for drug product release testing.

# 3.2.P.7 Container Closure System (Drug Product)

20 mL Type I borosilicate glass vials with 20 mm fluoropolymer-coated butyl rubber stoppers and aluminum flip-off seals. Container closure integrity verified per USP <1207>.

# 3.2.P.8 Stability (Drug Product)

Drug product stability studies per ICH Q1A/Q5C. Long-term (5C), accelerated (25C/60%RH), and stress (40C/75%RH). Proposed shelf-life of 24 months at 2-8 degrees C.

| Attribute | Condition | T=0 | 3M | 6M | 12M |
| --- | --- | --- | --- | --- | --- |
| Purity (SEC) | 5C | 97.2 | 97.0 | 96.8 | 96.5 |
| Potency | 5C | 100 | 99 | 98 | 97 |
| Subvisible Particles | 5C | 12 | 15 | 18 | 22 |