# Stability Study Report -- mAb-Stab-01

This stability study was conducted per ICH Q1A(R2) and ICH Q5C guidelines to support the proposed shelf-life of 24 months at 2-8 degrees C for mAb-Stab-01 drug product.

Shelf-life of 24 months is proposed based on long-term, accelerated, and stress stability data.

# 1. Long-Term Stability (5 degC)

Long-term stability study at 5 degC (refrigerated storage). Samples were tested at T=0, 3M, 6M, 9M, 12M, 18M, and 24M.

| Attribute | T=0 | 3M | 6M | 9M | 12M | 18M | 24M |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purity (SEC) | 99.2 | 99.1 | 99.0 | 98.8 | 98.6 | 98.3 | 98.0 |
| HMW (%) | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | 1.1 | 1.3 |
| Potency (%) | 102 | 101 | 101 | 100 | 100 | 99 | 98 |
| pH | 6.2 | 6.2 | 6.2 | 6.1 | 6.1 | 6.1 | 6.1 |
| Subvisible Particles | 12 | 15 | 18 | 20 | 22 | 25 | 28 |

# 2. Accelerated Stability (25 degC / 60% RH)

Accelerated stability study at 25 degC / 60% RH per ICH Q1A.

| Attribute | T=0 | 3M | 6M | 12M |
| --- | --- | --- | --- | --- |
| Purity (SEC) | 99.2 | 98.5 | 97.8 | 96.5 |
| HMW (%) | 0.5 | 1.0 | 1.8 | 2.8 |
| Potency (%) | 102 | 99 | 96 | 92 |
| Appearance | Clear | Clear | Clear | Slightly opalescent |

# 3. Stress Stability (40 degC / 75% RH)

Stress stability study at 40 degC / 75% RH. OOS observed for potency at 3M timepoint.

| Attribute | T=0 | 1M | 3M |
| --- | --- | --- | --- |
| Purity (SEC) | 99.2 | 96.1 | 91.5 |
| HMW (%) | 0.5 | 2.5 | 5.8 |
| Potency (%) | 102 | 88 | FAIL - OOS |

# 4. Summary and Conclusions

The long-term stability data at 5 degC supports a shelf-life of 24 months. Accelerated data at 25 degC / 60% RH shows acceptable degradation kinetics. Stress data at 40 degC / 75% RH demonstrates expected rapid degradation with an OOS event for potency at 3 months. All stability-indicating attributes monitored include purity, aggregation, potency, pH, and subvisible particles.